- UDP Broadcast(Spec43=1)

Remote Port=33583, Listen Port=33584

In same local-network, device will broad cast in port 33584, so listen in this port can known the ip of the devices.

Or send " UDP\t\r\n " in port 33583, to ask the device in local-network to broadcast immediately.

- TCP Connect(Spec43=1)

Create a TCP socket to 33581 of the device. Then control the device by this connection. There are many commands, we list most useful ones.

- TCP Connect(Spec43=2)

Create a TCP Server in 33582. Scales will connect to this server.

  - Get information of device

PC Send: "UPL\tINF\t\r\n"

PC Receive: (Please note all the \t )

DWL	INF

INA	GB2312	GB2312	0

INM	0023F0FFBD70	0023F0FFBD70

INF	JHScale	FactoryDevice	1	JB-xA CC003 V2.51A

END	INF	(END INF is end symbol, mean FINISH)

The first segment after INM is MAC

  - PC Send data

Refer to PLU.txt for sample, and you can just send the file directly.

You can just read TMS file from "TM-xA software" to know more details.

After you send all the data. Please DO NOT DISCONNECT immediately. The data may still on its way.

Do "Get information of device" first, if you receive any data, means all data have been dealled.

  - PC receive data

Send "UPL\tPLU\t\r\n" is ask device to upload all PLUs

Send "UPL\tPLU\t1000\t\r\n" is ask device to upload PLU whose ID=1000

Other data is similar.

  - Receive LOG

PC Send: "UPL\tREP\t-1\t\r\n" or PC Send: "UPL\tREP\t1\t\r\n"

Please note, "-1" means get all data FID>=1, even if the Log is been mark "Deleted". "1" means get all data FID>=1 only when it is not mark "Deleted"

PC Receive: (Please note all the \t )

DWL	REP

REP	14	7	17	10	43	46	137	1	0

RES	1	5,2	9999998	2	2	0,0	1,0	2	2	0,2					5,2	0	T-Sale

REE

REP	14	7	17	10	43	54	138	2	0

RES	1	2100,2	1	9	9	0,0	35,2	3	2	0,2					6000,2	0

REE

REP	14	7	17	10	43	59	139	3	0

RES	1	960,2	1	9	9	0,0	16,2	3	2	0,2					6000,2	0

REE

END	REP	(END REP is end symbol, mean FINISH)

The segment in them:

REP: Year, Month, Day, Hour, Minutes, Second, FID, SID, Salesman

RES: SubID, Money, PLU, Class, Dept, Cost, Amount, Unit(3=kg, 2=pcs), Type, Tax, Text1, Text2, Text 3, Text 4, UnitPrice, CoFID, PLU Name

All decimal is use a special express: "960,2=9.60", "6000,2=60.00"

  - Delete LOG

PC Send: "CLR\tREP\t1000\t\r\nCLR\tEND\t\r\n"

This command do not really delete the LOG, it just mark delete. All FID<=1000 are marked as deleted.

Type Information

0:Sale|1|1:BuyBack|2|2:Unconfirmed|3|3:Deleted|4|4:Hangup|5|5:Auto Mode|6|6:SP06|7|7:SP07|8|8:SP08|9|9:SP09|10|10:Inbound|11|11:Outbound|12|12:Stocktaking|13|13:Order|14|14:Spoilage|15|15:SP15|998|998:Unpacking(Increase)|999|999:Unpacking(Decre