| Name | Value | Description |
| --- | --- | --- |
| BeepMode | 0x00000000 0x00000001 0x00000002 | Off Beep Beep when touched Beep when released |
| BeepFrequency | 0x00000064 ~0x00000FA0 | Beep Frequency |
| BeepDuration | 0x00000019 ~0x000001F4 | Beep duration |
| MouseMode | 0x80000000 0x40000000 0x20000000 0x10000000 0x01000000 0x00000000 0x00000001 0x00000002 0x00000003 0x00000004 | make driver to report right button. enable auto right button feature. disable driver to report mouse event. Set this flag to make driver to by pass the software filter to get more fast response. Disable rotation support. Default value of mouse emulation mode. set the mouse emulation mode to be “Click on Touch”. set the mouse emulation mode to be “Click on Release”. set the mouse emulation mode to be “Click On Touch No Move”. set the mouse emulation mode to be “click On Release No Move”. |
| ConstDuration | 0x00000000 ~0x00000078 ( 0 ~ 120 ) | constant touch parameter which define the touch duration in milli-second for auto right click.( 3Sec = 120 * 0.025 ) |
| ConstRange | 0x00000020 ~0x000008C0 | constant touch parameter which define the touch range for auto right click. ( 1 pixel = 0x00000020 >> 5) |

If you want modify default setting value. You can modify “ reg.ini ” file.

Please refer reg.ini and this readme file.

For Windows driver 4.3.0 only.

Edge compensation parameter

| Name | Default value | Description |
| --- | --- | --- |
| lLeft lRight lTop lBottom lOffsetX lOffsetY ulMode | 110 (Range:50~150) 110 (Range:50~150) 110 (Range:50~150) 110 (Range:50~150) 0 0 0x00000001 | lLeft Edge compensation parameter in left side. lRight Edge compensation parameter in right side. lTop Edge compensation parameter in upper side lBottom Edge compensation parameter in bottom side lOffsetX Offset parameter in X direction. slider range of lOffsetX in Edge page: 0~40(percent of offset –20% ~ 20%) LOffsetX = ( slider value - 20 ) * 64 Ex:if you want to set X direction offset to –20% LOffsetX = ( 0 - 20 ) * 64 If you want to set of X direction offset to 20% LOffsetX = ( 40 - 20 ) * 64 lOffsetY Offset parameter in Y direction. slider range of lOffsetY in Edge page: 0~40(percent of offset –20% ~ 20%) LOffsetX = ( slider value - 20 ) * 64 Ex: if you want to set Y direction offset to –20% LOffsetY = ( 0 - 20 ) * 64 If you want to set Y direction offset to 20% LOffsetY = ( 40 - 20 ) * 64 ulMode Flag to enable/disable edge compensation. This flag must be either 0 or 1 |

Calibration Information data

| Name | Defualt value | Description |
| --- | --- | --- |
| lLLX lLLY lLRX lLRY lURX lURY lULX lULY | 102 102 1945 102 1945 1945 102 1945 | calibration parameter in X direction at lower left corner. calibration parameter in Y direction at lower left corner. calibration parameter in X direction at lower right corner. calibration parameter in Y direction at lower right corner. calibration parameter in X direction at upper right corner. calibration parameter in Y direction at upper right corner. calibration parameter in X direction at upper left corner. calibration parameter in Y direction at upper left corner. |

Linearization information data

| Name | Defualt value | Description |
| --- | --- | --- |
| LINZ_lLLX LINZ_lLLY LINZ_lLRX LINZ_lLRY LINZ_lURX LINZ_lURY LINZ_lULX LINZ_lULY LINZ_X1 LINZ_Y1 LINZ_X2 LINZ_Y2 LINZ_X3 LINZ_Y3 LINZ_X4 LINZ_Y4 LINZ_X5 LINZ_Y5 LINZ_X6 LINZ_Y6 LINZ_X7 LINZ_Y7 LINZ_X8 LINZ_Y8 LINZ_X9 LINZ_Y9 LINZ_X10 LINZ_Y10 LINZ_X11 LINZ_Y11 LINZ_X12 LINZ_Y12 LINZ_X13 LINZ_Y13 LINZ_X14 LINZ_Y14 LINZ_X15 LINZ_Y15 LINZ_X16 LINZ_Y16 LINZ_X17 LINZ_Y17 LINZ_X18 LINZ_Y18 LINZ_X19 LINZ_Y19 LINZ_X20 LINZ_Y20 LINZ_X21 LINZ_Y21 LINZ_X22 LINZ_Y22 LINZ_X23 LINZ_Y23 LINZ_X24 LINZ_Y24 LINZ_X25 LINZ_Y25 | 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 | calibration parameter in X direction at lower left corner. calibration parameter in Y direction at lower left corner. calibration parameter in X direction at lower right corner. calibration parameter in Y direction at lower right corner. calibration parameter in X direction at upper right corner. calibration parameter in Y direction at upper right corner. calibration parameter in X direction at upper left corner. calibration parameter in Y direction at upper left corner. Linearization information data. |

Linearization Type

| Name | Default Value | Description |
| --- | --- | --- |
| LinearizationType | 0x00000001 | 0x00000000 25 points calibration. 0x00000001 9 points calibration. |

Right button window display

| Name | Default Value | Description |
| --- | --- | --- |
| ButtonDisplay | 0x00000001 | 0x00000000 Hide right button window. 0x00000001 Display right button window. |

Double click setting

| Name | Default Value | Description |
| --- | --- | --- |
| DBCLICKSPEED | 600 ( Range : 300 ~ 900 ) | Set double click speed for system. |
| DBCLICKSIZE | 68 ( Range : 8 ~ 128 ) | Set double click area for system. |

Source of Calibration data

| Name | Default Value | Description |
| --- | --- | --- |
| CalSource | 0 | 0 : Read calibration data from registry. Write calibration data to registry. 1: Read calibration data from EEPROM. Write calibration data to EEPROM. |

Display setting

| Name | Default Value | Description |
| --- | --- | --- |
| uMonitorIndex uMonitorMode | 0x00000000 0x00000000 | Index to indicate which monitor to be mapped to. parameter to define the monitor mode. The mode can be the following combination: 0x10000000 : Set this flag to enable multiple monitor operation. 0x20000000 : Set this flag to enable split display function. 0x80000000 : Map to Main Monitor if only one display. If you want to set split display area, You must enable split display flag. Split display area : 0x00000000 Full screen or customized. 0x00000001 Upper screen. 0x00000002 Lower screen. 0x00000003 Right screen. 0x00000004 Left screen. 0x00000005 Quarter 1 0x00000006 Quarter 2 0x00000007 Quarter 3 0x00000008 Quarter 4 |
| MoniterArea.Left MoniterArea.Top MoniterArea.Right MoniterArea.Bottom WorkArea.Left WorkArea.Top WorkArea.Right WorkArea.Bottom VirtualDesktopArea.Left VirtualDesktopArea.Top VirtualDesktopArea.Right VirtualDesktopArea.Bottom | 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 0x00000000 | monitor location. the working area of the monitor. the system virtual desktop area. |